## DG-S1930K Upgrade Operation Guide

- Upgrade via Uboot

During the device restart process，press “ctrl+c” then select “0”

then select “1”

Keep the pc and the device on the same network segment, and enable the tftp server on the pc, then enter “the local ip”,means device ip address; enter “the remote ip”,means pc ip address; enter“the filename”，

S1930K_DGOS11.4(1)B70P18S1_12151903_middle.bin

Then select “Y” and then begin to upgrade

After upgrade，press“ctrl+z”

Then slecet 2, device bengin to  reload, after reload finished, show version detail

Then we upgrade the final software file with cli. Use command to upgrade with tftp: “upgrade download tftp://XXXX(pc ip)/

S1930K_DGOS11.4(1)B70P18S1_12151821_install.bin

And then device reload auto.

after reload finished, show version detail

Check the boot version, changed from 3.6.2(700101) to 3.6.2_123e3419(700101).

- Upgrade via CLI

Let's check the old version first. Use the command：show version detail

Then we upgrade the first software file with cli. Use command to upgrade with tftp: “upgrade download tftp://XXXX(pc ip)/

S1930K_DGOS11.4(1)B70P18S1_12151903_middle.bin

And then device reload auto.

after reload finished, show version detail

Then we upgrade the final software file with cli. Use command to upgrade with tftp: “upgrade download tftp://XXXX(pc ip)/

S1930K_DGOS11.4(1)B70P18S1_12151821_install.bin

And then device reload auto.

after reload finished, show version detail

Check the boot version, changed from 3.6.2(700101) to 3.6.2_123e3419(700101).

- Upgrade via cloud

Upload the version on the cloud

Check version first

Upgrade the first software: S1930K_DGOS11.4(1)B70P18S1_12151903_middle.bin

Check upgrade status

After upgrade, check the version

Upgrade the final software: S1930K_DGOS11.4(1)B70P18S1_12151821_install.bin

After upgrade, check the versión

Check the boot version, changed from 3.6.2(700101) to 3.6.2_123e3419(700101).

- Upgrade via WEB

Check version first

Upgrade the first software: S1930K_DGOS11.4(1)B70P18S1_12151903_middle.bin

After upgrade, check the versión

Upgrade the final software: S1930K_DGOS11.4(1)B70P18S1_12151821_install.bin

After upgrade, check the versión

Check the boot version, changed from 3.6.2(700101) to 3.6.2_123e3419(700101).